:8042:keyboard controller

^8042 Commands Related to PC Systems  (Port 64h)

%	Command 		   Description

20   Read 8042 Command Byte: current 8042 command byte is placed

in port 60h.

60   Write 8042 Command Byte: next data byte written to port 60h is

placed in 8042 command register.	Format:

|7|6|5|4|3|2|1|0|	8042 Command Byte

| | | | | | | |____ 1=enable output register full interrupt

| | | | | | |_____ should be 0

| | | | | |______ 1=set status register system, 0=clear

| | | | |_______ 1=override keyboard inhibit, 0=allow inhibit

| | | |________ disable keyboard I/O by driving clock line low

| | |_________ disable auxiliary device, drives clock line low

| |__________ IBM scancode translation 0=AT, 1=PC/XT

|___________ reserved, should be 0

A4   Password Installed Test: returned data can be read

from port 60h;  FA=password installed, F1=no password

A5   Load Security: bytes written to port 60h will be read

until a null (0) is found.

A6   Enable Security: works only if a password is already loaded

A7   Disable Auxiliary Interface: sets Bit 5 of command register

stopping auxiliary I/O by driving the clock line low

A8   Enable Auxiliary Interface: clears Bit 5 of command register

A9   Auxiliary Interface Test: clock and data lines are tested;

results placed at port 60h are listed below:

00  no error

01  keyboard clock line is stuck low

02  keyboard clock line is stuck high

03  keyboard data line is stuck low

04  keyboard data line is stuck high

AA   Self Test: diagnostic result placed at port 60h, 55h=OK

AB   Keyboard Interface Test:	clock and data lines are tested;

results placed at port 60h are listed above with command A9

AC   Diagnostic Dump: sends 16 bytes of 8042's RAM, current input

port state, current output port state and 8042 program status

word to port 60h in scan-code format.

AD   Disable Keyboard Interface: sets Bit 4 of command register

stopping keyboard I/O by driving the clock line low

AE   Enable Keyboard Interface: clears Bit 4 of command register

enabling keyboard interface.

C0   Read Input Port: data is read from its input port (which is

inaccessible to the data bus) and written to output register

at port 60h;  output register should be empty before call.

|7|6|5|4|3-0|  8042 Input Port

| | | | |____ undefined

| | | |_____ 1=enable 2nd 256K of motherboard RAM, 0=disable

| | |______ 1=manufacturing jumper not installed, 0=installed

| |_______ 1=primary display is MDA, 0=primary display is CGA

|________ 1=keyboard not inhibited, 0=keyboard inhibited

C1   Poll Input Port Low Bits: Bits 0-3 of port 1 placed in

status Bits 4-7

C2   Poll Input Port High Bits: Bits 4-7 of port 1 placed in

status Bits 4-7

D0   Read Output Port: data is read from 8042 output port (which is

inaccessible to the data bus) and placed in output register;

the output register should be empty.  (see command D1 below)

D1   Write Output Port: next byte written to port 60h is placed in

the 8042 output port (which is inaccessible to the data bus)

|7|6|5|4|3|2|1|0|  8042 Output Port

| | | | | | | |____ system reset line

| | | | | | |_____ gate A20

| | | | |_|______ undefined

| | | |_________ output buffer full

| | |__________ input buffer empty

| |___________ keyboard clock (output)

|____________ keyboard data (output)

D2   Write Keyboard Output Register: on PS/2 systems the next data

byte written to port 60h input register is written to port 60h

output register as if initiated by a device; invokes interrupt

if enabled

D3   Write Auxiliary Output Register: on PS/2 systems the next data

byte written to port 60h input register is written to port 60h

output register as if initiated by a device; invokes interrupt

if enabled

D4   Write Auxiliary Device: on PS/2 systems the next data byte

written to input register a port at 60h is sent to the

auxiliary device

E0   Read Test Inputs: 8042 reads its T0 and T1 inputs; data is

placed in output register;  Bit 0 is T0, Bit 1 is T1:

|1|0|  Test Input Port Bits

| |____ keyboard clock

|_____ keyboard data

Fx   Pulse Output Port: Bits 0-3 of the 8042 output port can be

pulsed low for 6 �s;  Bits 0-3 of command indicate which

Bits should be pulsed; 0=pulse, 1=don't pulse; pulsing

Bit 0 results in CPU reset since it is connected to system

reset line.

- PC systems previous to the AT use the 8255 PPI as a keyboard

controller and use the keyboard's internal 8048.

- the keyboard's internal controller buffers up to 16 bytes of

make/break code information. This is common among all PC systems

and shouldn't be confused with the (32 byte) keyboard buffer

maintained by the BIOS.