:KB flags:keyboard flags

^Keyboard Flags Bytes 0 and 1

|7|6|5|4|3|2|1|0|  40:17  Keyboard Flags Byte 0

| | | | | | | |____ right shift key depressed

| | | | | | |_____ left shift key depressed

| | | | | |______ CTRL key depressed

| | | | |_______ ALT key depressed

| | | |________ scroll-lock is active

| | |_________ num-lock is active

| |__________ caps-lock is active

|___________ insert is active

|7|6|5|4|3|2|1|0|  40:18  Keyboard Flags Byte 1

| | | | | | | |____ left CTRL key depressed

| | | | | | |_____ left ALT key depressed

| | | | | |______ system key depressed and held

| | | | |_______ suspend key has been toggled

| | | |________ scroll lock key is depressed

| | |_________ num-lock key is depressed

| |__________ caps-lock key is depressed

|___________ insert key is depressed

^Keyboard Flags Bytes 2 and 3

|7|6|5|4|3|2|1|0|  40:97  LED Indicator Flags

| | | | | | | |____ scroll lock indicator

| | | | | | |_____ num-lock indicator

| | | | | |______ caps-lock indicator

| | | | |_______ circus system indicator

| | | |________ ACK received

| | |_________ re-send received flag

| |__________ mode indicator update

|___________ keyboard transmit error flag

|7|6|5|4|3|2|1|0|  40:96  Keyboard Mode/Type

| | | | | | | |____ last code was the E1 hidden code

| | | | | | |_____ last code was the E0 hidden code

| | | | | |______ right CTRL key depressed

| | | | |_______ right ALT key depressed

| | | |________ 101/102 enhanced keyboard installed

| | |_________ force num-lock if Rd ID & KBX

| |__________ last char was first ID char

|___________ read ID in process