:make codes:break codes

^INT 9 - Hardware Keyboard Make/Break Codes

%	Key	     Make  Break		Key    Make  Break

Backspace   0E    8E			F1	3B    BB

Caps Lock   3A    BA			F2	3C    BC

Enter	      1C    9C			F3	3D    BD

Esc	      01    81			F4	3E    BE

Left Alt    38    B8			F7	41    C1

Left Ctrl   1D    9D			F5	3F    BF

Left Shift  2A    AA			F6	40    C0

Num Lock    45    C5			F8	42    C2

Right Shift 36    B6			F9	43    C3

Scroll Lock 46    C6			F10	44    C4

Space	      39    B9			F11	57    D7

Sys Req(AT) 54    D4			F12	58    D8

Tab	      0F    8F

%		    Keypad Keys	           Make   Break

Keypad 0  (Ins)		52	D2

Keypad 1  (End) 		4F	CF

Keypad 2  (Down arrow)	50	D0

Keypad 3  (PgDn)		51	D1

Keypad 4  (Left arrow)	4B	CB

Keypad 5			4C	CC

Keypad 6  (Right arrow)	4D	CD

Keypad 7  (Home)		47	C7

Keypad 8  (Up arrow)	48	C8

Keypad 9  (PgUp)		49	C9

Keypad .  (Del) 		53	D3

Keypad *  (PrtSc)		37	B7

Keypad -			4A	CA

Keypad +			4E	CE

%	       Key    Make  Break	       Key    Make  Break

A      1E    9E 		  N      31    B1

B      30    B0 		  O      18    98

C      2E    AE 		  P      19    99

D      20    A0 		  Q      10    90

E      12    92 		  R      13    93

F      21    A1 		  S      1F    9F

G      22    A2 		  T      14    94

H      23    A3 		  U      16    96

I      17    97 		  V      2F    AF

J      24    A4 		  W      11    91

K      25    A5 		  X      2D    AD

L      26    A6 		  Y      15    95

M      32    B2 		  Z      2C    AC

%	       Key    Make  Break	       Key    Make  Break

1      02    82 		  -      0C    8C

2      03    83 		  =      0D    8D

3      04    84 		  [      1A    9A

4      05    85 		  ]      1B    9B

5      06    86 		  ;      27    A7

6      07    87 		  '      28    A8

7      08    88 		  `      29    A9

8      09    89 		  \      2B    AB

9      0A    8A 		  ,      33    B3

0      0B    8B 		  .      34    B4

/      35    B5

^Enhanced Keyboard Keys (101/102 keys)

%	Control Keys	  	Make	  		Break

Alt-PrtSc (SysReq)  	54		  	D4

Ctrl-PrtSc		  	E0 37 	  	E0 B7

Enter			  	E0 1C 	  	E0 9C

PrtSc			  	E0 2A E0 37		E0 B7 E0 AA

Right Alt		  	E0 38 	  	E0 B8

Right Ctrl		  	E0 1D 	  	E0 9D

Shift-PrtSc		  	E0 37 	  	E0 B7

/			  	E0 35 	  	E0 B5

Ctrl-Pause (Ctrl-Break)	E0 46 E0 C6  (not typematic)

Pause			  	E1 1D 45 E1 9D C5  (not typematic)

- Keys marked as "not typematic" generate one stream of bytes

without corresponding break scan code bytes (actually the

break codes are part of the make code).

%			 Normal Mode or

%			 Shift w/Numlock

%	Key		 Make	 	 Break   	   /------ Numlock on ------\

%					      		Make	    	    Break

Del		 E0 53	 E0 D3	   E0 2A E0 53	 E0 D3 E0 AA

Down arrow	 E0 50	 E0 D0	   E0 2A E0 50	 E0 D0 E0 AA

End		 E0 4F	 E0 CF	   E0 2A E0 4F	 E0 CF E0 AA

Home		 E0 47	 E0 C7	   E0 2A E0 47	 E0 C7 E0 AA

Ins		 E0 52	 E0 D2	   E0 2A E0 52	 E0 D2 E0 AA

Left arrow	 E0 4B	 E0 CB	   E0 2A E0 4B	 E0 CB E0 AA

PgDn		 E0 51	 E0 D1	   E0 2A E0 51	 E0 D1 E0 AA

PgUp		 E0 49	 E0 C9	   E0 2A E0 49	 E0 C9 E0 AA

Right arrow	 E0 4D	 E0 CD	   E0 2A E0 4D	 E0 CD E0 AA

Up arrow	 E0 48	 E0 C8	   E0 2A E0 48	 E0 C8 E0 AA

%	Key	      /--Left Shift Pressed--\	/--Right Shift Pressed--\

%			   Make	     Break	         Make	    Break

Del	      E0 AA E0 53   E0 D3 E0 2A	E0 B6 E0 53	 E0 D3 E0 36

Down arrow  E0 AA E0 50   E0 D0 E0 2A	E0 B6 E0 50	 E0 D0 E0 36

End	      E0 AA E0 4F   E0 CF E0 2A	E0 B6 E0 4F	 E0 CF E0 36

Home	      E0 AA E0 47   E0 C7 E0 2A	E0 B6 E0 47	 E0 C7 E0 36

Ins	      E0 AA E0 52   E0 D2 E0 2A	E0 B6 E0 52	 E0 D2 E0 36

Left arrow  E0 AA E0 4B   E0 CB E0 2A	E0 B6 E0 4B	 E0 CB E0 36

PgDn	      E0 AA E0 51   E0 D1 E0 2A	E0 B6 E0 51	 E0 D1 E0 36

PgUp	      E0 AA E0 49   E0 C9 E0 2A	E0 B6 E0 49	 E0 C9 E0 36

Right arrow E0 AA E0 4D   E0 CD E0 2A	E0 B6 E0 4D	 E0 CD E0 36

Up arrow    E0 AA E0 48   E0 C8 E0 2A	E0 B6 E0 48	 E0 C8 E0 36

/	      E0 AA E0 35   E0 B5 E0 2A	E0 B6 E0 35	 E0 B5 E0 36

- The PS/2 models have three make/break scan code sets.  The first

set matches the PC & XT make/break scan code set and is the one

listed here.	Scan code sets are selected by writing the value F0

to the keyboard via the ~8042~ (port 60h).  The following is a brief

description of the scan code sets (see the PS/2 Technical Reference

manuals for more information on scan code sets 2 and 3):

*  set 1, each key has a base scan code.  Some keys generate

extra scan codes to generate artificial shift states.  This

is similar to the standard scan code set used on the PC and XT.

*  set 2, each key sends one make scan code and two break scan

codes bytes (F0 followed by the make code).  This scan code

set is available on the IBM AT also.

*  set 3, each key sends one make scan code and two break scan

codes bytes (F0 followed by the make code) and no keys are

altered by Shift/Alt/Ctrl keys.

*  typematic scan codes are the same as the make scan code

- Some Tandy 1000's do not handle Alt key combinations when multiple

shift keys are pressed.  The Alt-Shift-H combination loses the Alt.

- extended keys like (F11, F12) can only be read with systems that

have extended keyboard BIOS support (or ~INT 9~ extensions);  to

read these special keys on these systems ~INT 16,10~ must be used.