# PC Keyboard Scan Codes

Altek Instruments have designed and built a general purpose, low cost PC interface which generates Keyboard Scancodes. Any key or combination of keys may be emulated thus enabling any applications program to interact with RS232 peripheral devices. We call it The Mule™.

Altek Instruments Ltd have a range of products designed for the data collection/automatic ID industries.

## Overview

The PC keyboard interface is designed so the system software has maximum flexibility in defining certain keyboard operations. This is accomplished by having the keyboard return scancodes rather than ASCII codes. Each key generates a 'make' scancode when pressed and a 'break' scancode when released. The computer system interprets the scancodes to determine what operation it is to perform. For historical compatibility reasons computers can employ different sets of scancodes for different purposes. Earlier PC computers tend to use the XT scancode set. Current PC computers tend to use the AT scancode set. The MF1, MF2 and MF3 sets are alternatives, similar in many respects to the XT and AT sets. The table below shows the scancodes generated when a key is pressed- the 'make' code. The 'break' code may be derived from the 'make' code as described later. Notice that scancodes may consist of a single or several bytes.

Many keyboard scancode tables published are not completely accurate. Confusion arises between scancodes issued by the keyboard hardware and the scancodes appearing in the keyboard buffer. The scancodes presented here are those actually sent by the keyboard hardware. The computer BIOS may translate the code for compatibility reasons. For example, the XT scancode set was first used in the original IBM PC computer. Other IBM computers and terminals used a different scancode set. Apparently IBM tried to bring the PC into line with its other products when the IBM PC/AT was launched. The keyboard for that machine used a scancode set virtually identical to other IBM machines. However to maintain compatibility with earlier PC software the BIOS for the AT machines converted the AT scancodes into the earlier XT scancodes. At the current time (June 1995) this system of code conversion in the BIOS persists. The code you read from the keyboard buffer is not necessarily the same code transmitted from the keyboard.

## How to Calculate the Scancode

- Use the keyboard sketches to determine the 'Key Number'. Note that the 102 key keyboard shown is the keyboard used in the UK. In the USA and some other countries a 101 key keyboard is used- there are minor scancode differences- read the table notes.

- Lookup the 'Key number' in the table and read of the 'Make'scancode. Note that the scancode differs depending on the scancode set currently in use. Note also that some scancodes consist of more than 1 byte.

- Calculate the 'break' scancode as detailed below

## 'Break' scancodes for PC/XT computers

The 'break' code for XT style computers is the 'Make' code OR'd with the hexadecimal value 80. (If you are unfamiliar with hexadecimal numbers then simply change the first character of the scancode thus: Change 0 to 8, 1 to 9, 2 to A, 3 to B, 4 to C, 5 to D, 6 to E, 7 to F)

For example the scancodes generated by pressing and releasing the Escape key (Key Number 110) are 01 81

## 'Break' scancodes for PC/AT computers

Most PC's made since about 1989 use keyboards that generate AT scancodes (Though the codes may be converted to XT scancodes in the BIOS). The 'break' code for AT class scancodes is simply the 'make' code preceded by hex F0. For example the scancodes generated when the Escape key is pressed and released are 76 F0 76.

## Special scancodes of potential interest to Altek Mule(tm) users

If the keyboard's buffer overflows it sends a special scancode to the computer. This generally causes the computer to sound a warning beep. Altek Mule(tm) users may issue this code if they want to sound the beep to draw the operators attention. For PC computers using XT scancodes the code is hex FF. For computers using AT scancodes the code is 00.

## PC Keyboard, 102 key

## PC Keyboard, 84 key

## Scancode Table

### Table Notes

- The Scancode values in this table are hexadecimal

- Some key numbers are unallocated or reserved by IBM

- * Only keyboards with 101 keys- USA keyboard (and others)

- ** Only keyboards with 102 keys- UK keyboard (and others)

| Scancode Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Key Num | XT | AT | MF1 | MF2 | MF3 |
| 1 | 29 | 0E | 29 | 0E | 0E |
| 2 | 02 | 16 | 02 | 16 | 16 |
| 3 | 03 | 1E | 03 | 1E | 1E |
| 4 | 04 | 26 | 04 | 26 | 26 |
| 5 | 05 | 25 | 05 | 25 | 25 |
| 6 | 06 | 2E | 06 | 2E | 2E |
| 7 | 07 | 36 | 07 | 36 | 36 |
| 8 | 08 | 3D | 08 | 3D | 3D |
| 9 | 09 | 3E | 09 | 3E | 3E |
| 10 | 0A | 46 | 0A | 46 | 46 |
| 11 | 0B | 45 | 0B | 45 | 45 |
| 12 | 0C | 4E | 0C | 4E | 4E |
| 13 | 0D | 55 | 0D | 55 | 55 |
| 15 | 0E | 66 | 0E | 66 | 66 |
| 16 | 0F | 0D | 0F | 0D | 0D |
| 17 | 10 | 15 | 10 | 15 | 15 |
| 18 | 11 | 1D | 11 | 1D | 1D |
| 19 | 12 | 24 | 12 | 24 | 24 |
| 20 | 13 | 2D | 13 | 2D | 2D |
| 21 | 14 | 2C | 14 | 2C | 2C |
| 22 | 15 | 35 | 15 | 35 | 35 |
| 23 | 16 | 3C | 16 | 3C | 3C |
| 24 | 17 | 43 | 17 | 43 | 43 |
| 25 | 18 | 44 | 18 | 44 | 44 |
| 26 | 19 | 4D | 19 | 4D | 4D |
| 27 | 1A | 54 | 1A | 54 | 54 |
| 28 | 1B | 5B | 1B | 5B | 5B |
| * 29 | 2B | 5D | 2B | 5D | 5C |
| 30 | 3A | 58 | 3A | 58 | 14 |
| 31 | 1E | 1C | 1E | 1C | 1C |
| 32 | 1F | 1B | 1F | 1B | 1B |
| 33 | 20 | 23 | 20 | 23 | 23 |
| 34 | 21 | 2B | 21 | 2B | 2B |
| 35 | 22 | 34 | 22 | 34 | 34 |
| 36 | 23 | 33 | 23 | 33 | 33 |
| 37 | 24 | 3B | 24 | 3B | 3B |
| 38 | 25 | 42 | 25 | 42 | 42 |
| 39 | 26 | 4B | 26 | 4B | 4B |
| 40 | 27 | 4C | 27 | 4C | 4C |
| 41 | 28 | 52 | 28 | 52 | 52 |
| ** 42 | 2B | 5D | 2B | 5D | 53 |
| 43 | 1C | 5A | 1C | 5A | 5A |
| 44 | 2A | 12 | 2A | 12 | 12 |
| ** 45 | 56 | 61 | 56 | 61 | 13 |
| 46 | 2C | 1A | 2C | 1A | 1A |
| 47 | 2D | 22 | 2D | 22 | 22 |
| 48 | 2E | 21 | 2E | 21 | 21 |
| 49 | 2F | 2A | 2F | 2A | 2A |
| 50 | 30 | 32 | 30 | 32 | 32 |
| 51 | 31 | 31 | 31 | 31 | 31 |
| 52 | 32 | 3A | 32 | 3A | 3A |
| 53 | 33 | 41 | 33 | 41 | 41 |
| 54 | 34 | 49 | 34 | 49 | 49 |
| 55 | 35 | 4A | 35 | 4A | 4A |
| 57 | 36 | 59 | 36 | 59 | 59 |
| 58 | 1D | 14 | 1D | 14 | 11 |
| 60 | 38 | 11 | 38 | 11 | 19 |
| 61 | 39 | 29 | 39 | 29 | 29 |
| 62 | E038 | E011 | E038 | E011 | 39 |
| 64 | E01D | E014 | E01D | E014 | 58 |
| 75 | E052 | E070 | E052 | E070 | 67 |
| 76 | E053 | E071 | E053 | E071 | 64 |
| 79 | E04B | E06B | E04B | E06B | 61 |
| 80 | E047 | E06C | E047 | E06C | 6E |
| 81 | E04F | E069 | E04F | E069 | 65 |
| 83 | E048 | E075 | E048 | E075 | 63 |
| 84 | E050 | E072 | E050 | E072 | 60 |
| 85 | E049 | E07D | E049 | E07D | 6F |
| 86 | E051 | E07A | E051 | E07A | 6D |
| 89 | E04D | E074 | E04D | E074 | 6A |
| 90 | 45 | 77 | 45 | 77 | 76 |
| 91 | 47 | 6C | 47 | 6C | 6C |
| 92 | 4B | 6B | 4B | 6B | 6B |
| 93 | 4F | 69 | 4F | 69 | 69 |
| 95 | E035 | E04A | E035 | E04A | 77 |
| 96 | 48 | 75 | 48 | 75 | 75 |
| 97 | 4C | 73 | 4C | 73 | 73 |
| 98 | 50 | 72 | 50 | 72 | 72 |
| 99 | 52 | 70 | 52 | 70 | 70 |
| 100 | 37 | 7C | 37 | 7C | 7E |
| 101 | 49 | 7D | 49 | 7D | 7D |
| 102 | 4D | 74 | 4D | 74 | 74 |
| 103 | 51 | 7A | 51 | 7A | 7A |
| 104 | 53 | 71 | 53 | 71 | 71 |
| 105 | 4A | 7B | 4A | 7B | 84 |
| 106 | 4E | 79 | 4E | 79 | 7C |
| 108 | E01C | E05A | E01C | E05A | 79 |
| 110 | 01 | 76 | 01 | 76 | 08 |
| 112 | 3B | 05 | 3B | 05 | 07 |
| 113 | 3C | 06 | 3C | 06 | 0F |
| 114 | 3D | 04 | 3D | 04 | 17 |
| 115 | 3E | 0C | 3E | 0C | 1F |
| 116 | 3F | 03 | 3F | 03 | 27 |
| 117 | 40 | 0B | 40 | 0B | 2F |
| 118 | 41 | 83 | 41 | 83 | 37 |
| 119 | 42 | 0A | 42 | 0A | 3F |
| 120 | 43 | 01 | 43 | 01 | 47 |
| 121 | 44 | 09 | 44 | 09 | 4F |
| 122 | 57 | 78 | 57 | 78 | 56 |
| 123 | 58 | 07 | 58 | 07 | 5E |
| 124 | E02AE037 | E012E07C | E02AE037 | E012E07C | 57 |
| 125 | 46 | 7E | 46 | 7E | 5F |
| 126 | E11D45E1 9DC5 | 1477F014 F077 | E11D45E1 9DC5 | E11477E1 F014F077 | 62 |

## Windows 95 Update

The Windows 95 keyboard has 3 extra keys. They are the two 'Flying Windows' keys and the 'Pop Up Menu' key. These keys may be emulated by The Mule in Scancode Mode just like any other key. The appropriate 'make' and 'break' scancodes are shown below...

|  | Make | Break |
| --- | --- | --- |
| Left Flying Window | E0 1F | E0 F0 1F |
| Right Flying Window | E0 27 | E0 F0 27 |
| Pop Up Menu | E0 2F | E0 F0 2F |